Title: SOS Emergency Alert System

Aim:
To design and develop a portable SOS emergency alert system that can send location-based SMS alerts, make emergency calls, and provide panic indication using a buzzer without relying on internet connectivity.

INTRODUCTION

1.1 Rationale

In recent years, personal safety has become a critical concern, especially in emergency situations where immediate communication is required. Many existing solutions depend heavily on smartphones and internet connectivity, which may not always be reliable or accessible during critical moments.

The proposed system focuses on developing a portable SOS emergency alert device that operates independently of internet services. By using GSM communication, the system ensures that emergency alerts can be sent quickly and reliably, even in low-network conditions.

The device integrates essential functionalities such as location sharing using GPS, emergency SMS alerts, calling capability, and a panic mode with an audible buzzer, making it a comprehensive safety solution. The system is designed to be simple, fast, and user-friendly, allowing users to trigger emergency actions with minimal effort.

Additionally, the project emphasizes reliability over complexity, ensuring that the core emergency features function effectively in real-world scenarios. This makes the device suitable for practical use in situations where immediate assistance is required.

1.2 Problem Definition and Proposed System

In today’s fast-paced world, individuals may encounter emergency situations such as accidents, health issues, or personal threats where immediate communication is essential. Existing safety solutions often rely on smartphones, mobile applications, and internet connectivity, which may not always be accessible, responsive, or reliable during critical moments. One of the major problems with current systems is the delay in sending alerts, dependency on user interaction with complex interfaces, and reliance on continuous internet connectivity. In many real-life situations, users may not have the time or ability to navigate through applications to seek help.

To address these challenges, this project proposes a hardware-based SOS emergency alert system that enables quick and direct communication using minimal user interaction. The system is designed to function independently of internet services by utilizing a GSM module for communication and a GPS module for location detection.

The proposed system allows the user to:

- Send an SOS alert message with location using a single button press

- Activate a panic mode with continuous buzzer alert for nearby attention

- Initiate an emergency call to a predefined contact

The device ensures that emergency alerts are delivered quickly, reliably, and with accurate location information, making it suitable for real-world applications. Additionally, although advanced features like real-time tracking using cloud platforms (Firebase) were explored during development, they were not implemented in the final system due to stability and integration challenges. The focus was therefore shifted to ensuring a robust and dependable core system.

1.3 System Overview

The proposed system is a portable SOS emergency alert device designed using an ESP32 microcontroller, integrated with GPS and GSM modules to provide fast and reliable emergency communication.

The ESP32 acts as the central controller of the system, managing input from buttons and coordinating communication between the GPS and GSM modules. The NEO-6M GPS module is responsible for acquiring the user’s geographical location in terms of latitude and longitude. This data is then processed and converted into a Google Maps link, which is included in the emergency message.

The SIM800L GSM module is used to send SMS alerts and make emergency calls to predefined contacts. This ensures that the system works independently of internet connectivity and can function in areas with basic cellular network availability.

The system provides three main functionalities:

- SOS Mode: Triggered by a short press of the SOS button, it sends an emergency message along with the user’s location.

- Panic Mode: Activated by a long press of the SOS button, it sends an alert message and activates a continuous buzzer to attract nearby attention.

- Calling Feature: A dedicated button allows the user to initiate and terminate an emergency call.

An active buzzer is used to provide audio feedback and alert signals during operation, especially in panic situations. The entire system is powered using a portable power supply, making it suitable for real-world usage. The design focuses on simplicity, reliability, and quick response, ensuring that users can seek help with minimal effort during emergencies.

1.4 Report Organization

This report is organized into multiple sections to provide a clear understanding of the project.

Chapter 1 presents the introduction, including the rationale, problem definition, and system overview.

Chapter 2 covers the literature survey, which discusses existing systems and their limitations.

Chapter 3 explains the requirements and system design, including hardware components, system architecture, and circuit design.

Chapter 4 describes the implementation details, including hardware setup and software logic.

Chapter 5 presents the expected outcomes and features of the system.

Chapter 6 includes the appendix, which contains supporting materials such as code snippets and system outputs.

Chapter 7 provides the conclusion and future scope of the project.

Chapter 8 lists the references and bibliography used in this project.

LITERATURE SURVEY

The concept of emergency alert systems has been widely explored in recent years, especially with the rise of IoT and mobile technologies. Various solutions have been developed to enhance personal safety and provide quick assistance during emergencies. Many existing systems are based on smartphone applications, where users can send alerts, share their location, or trigger emergency notifications through mobile apps. These applications often use internet connectivity and cloud platforms such as Firebase to enable real-time tracking and communication. While these systems provide advanced features, they have certain limitations, including dependency on internet access, higher battery consumption, and the need for user interaction with complex interfaces. Another category of systems involves wearable devices and IoT-based solutions, which integrate sensors, GPS modules, and communication units to detect and report emergency situations. Some systems use real-time tracking and continuously update location data to cloud servers. However, these solutions often face challenges such as increased system complexity, higher cost, and reliability issues due to continuous data transmission. Several projects have also utilized GSM-based communication systems, where SMS alerts are sent to predefined contacts during emergencies. These systems are considered more reliable in areas with limited internet connectivity, but they may lack advanced features like continuous tracking and remote monitoring. In this project, an attempt was made to implement real-time tracking using Firebase, where location data would be continuously updated and monitored. However, due to integration challenges between hardware components (ESP32, GPS, GSM) and network-related issues, this approach resulted in instability and reduced system performance. Therefore, the proposed system adopts a hybrid approach focused on reliability, using GPS for location detection and GSM for communication, without relying on continuous internet connectivity. This ensures that the system remains simple, efficient, and dependable in real-world emergency situations.

REQUIREMENT AND DESIGNING

3.1 Analysis

3.1.1 Requirement Analysis

The requirement analysis phase focuses on identifying the essential needs and functionalities of the proposed SOS emergency alert system. The primary objective of the system is to provide a fast, reliable, and easy-to-use emergency communication mechanism.

The system must be capable of sending an emergency alert message along with the user’s location to predefined contacts. It should also allow the user to initiate an emergency call and activate a panic mode with an audible buzzer for nearby attention. Since emergency situations may occur in areas with limited or no internet connectivity, the system is designed to function independently of internet services by using GSM-based communication. This ensures that alerts can be transmitted even in low-network conditions. The device should be portable, low-cost, and power-efficient, making it suitable for real-world usage. Additionally, the user interface must be simple, allowing users to trigger actions using minimal input, such as button presses. During development, advanced features like real-time tracking using cloud platforms were considered. However, based on system constraints and reliability concerns, the focus was shifted toward ensuring stable and efficient core functionalities.

3.1.2 Functional Requirements

Functional requirements define the core operations that the system must perform. The proposed SOS emergency alert system includes the following functionalities:

- SOS Alert Functionality
The system should send an emergency SMS to predefined contacts when the SOS button is pressed. The message must include a predefined alert text along with the user’s location in the form of a Google Maps link.

- Location Acquisition
The system should obtain the user’s current latitude and longitude using the GPS module and process this data to generate a usable location link.

- Panic Mode Activation
The system should activate a panic mode when the SOS button is pressed for a longer duration. In this mode, an emergency message is sent, and a continuous buzzer alert is generated to attract nearby attention.

- Emergency Calling Feature
The system should allow the user to initiate an emergency call to a predefined contact using a dedicated button. It should also allow the user to terminate the call when required.

- Audio Feedback System
The buzzer should provide feedback for different actions such as SOS activation, panic mode confirmation, and system responses.

- Button Input Handling
The system should correctly differentiate between short press and long press actions of the SOS button and respond accordingly.

- System Initialization
Upon powering ON, the system should initialize all modules, including GPS and GSM, and prepare them for operation.

3.1.3 Non-Functional Requirements

Non-functional requirements define the quality attributes and constraints of the system. The proposed SOS emergency alert system is designed with the following non-functional requirements:

- Reliability
The system must operate reliably during emergency situations. It should ensure that SMS alerts and calls are executed successfully without failure.

- Performance
The system should respond quickly to user inputs, such as button presses, and perform actions like sending SMS and activating the buzzer with minimal delay.

- Usability
The device should be simple and easy to use, allowing users to trigger emergency actions with minimal effort and without requiring technical knowledge.

- Portability
The system should be compact and portable so that it can be easily carried and used in real-world situations.

- Power Efficiency
The device should consume minimal power to ensure longer battery life, especially during continuous operation in emergency conditions.

- Scalability
The system should allow future enhancements, such as integration with mobile applications or cloud platforms, without major redesign.

- Maintainability
The system should be easy to maintain and modify, including updating contact numbers or improving features.

3.2 Components Used

The proposed SOS emergency alert system is built using a combination of hardware components that work together to provide reliable emergency communication and alert functionality. The details of each component are as follows:

3.2.1 ESP32 Development Board

The ESP32 development board acts as the main controller of the system. It is responsible for processing inputs from buttons, controlling the buzzer, and managing communication between the GPS and GSM modules. It provides multiple GPIO pins and supports serial communication, which allows it to interface efficiently with external modules. The ESP32 ensures smooth coordination of all system operations.

3.2.2 NEO-6M GPS Module

The NEO-6M GPS module is used to obtain the geographical location of the user in terms of latitude and longitude.

It communicates with the ESP32 using serial communication and provides accurate positioning data. This data is processed by the system to generate a Google Maps link, which is included in the emergency SMS.

3.2.3 SIM800L GSM Module

The SIM800L GSM module is used for communication purposes, including sending SMS alerts and making emergency calls.

It operates using a SIM card and communicates with the ESP32 through AT commands over serial communication. The module ensures that emergency messages are delivered even in the absence of internet connectivity.

3.2.4 Push Buttons

Two push buttons are used for user interaction:

- SOS Button: Used to trigger emergency actions. A short press sends an SOS message, while a long press activates panic mode.

- Call Button: Used to initiate and terminate emergency calls.

These buttons provide a simple and effective way for the user to interact with the system.

3.2.5 Active Buzzer

An active buzzer is used to provide audio feedback and alerts.

It produces sound signals to indicate system actions such as SOS activation, panic mode, and confirmation of operations. In panic mode, it generates a continuous alert to attract nearby attention.

3.2.6 Power Supply System

The system is powered using a 3.7V Lithium-ion battery (18650) along with a power bank.

3.2.7 Connecting Wires and Enclosure

Jumper wires (male-male and male-female) are used to establish connections between components. A simple enclosure is used to hold all components together, making the device portable and suitable for practical usage.

3.3 Circuit Diagram

The circuit diagram of the SOS emergency alert system represents the interconnection between the ESP32 microcontroller, GPS module, GSM module, buttons, buzzer, and power supply components.

The ESP32 serves as the central unit, interfacing with all input and output devices. The NEO-6M GPS module is connected to the ESP32 via UART communication, where its TX pin is connected to the RX pin of the ESP32, allowing location data to be received.

Similarly, the SIM800L GSM module is connected using serial communication (UART). The TX and RX pins of the GSM module are connected to the corresponding RX and TX pins of the ESP32, enabling the transmission of AT commands for sending SMS and making calls.

The SOS button and call button are connected to the GPIO pins of the ESP32 and configured as input devices. These buttons allow the user to trigger different functionalities based on the duration of the press.

The active buzzer is connected to a GPIO pin of the ESP32 and acts as an output device, providing audio alerts during system operations, especially in panic mode.

The power supply system is designed to ensure stable operation of all components. A 3.7V Li-ion battery is used as the primary power source. Proper grounding is maintained across all components to ensure stable and noise-free operation of the system.

3.4 System Architecture

The system architecture of the SOS emergency alert device represents the overall structure and flow of operations between different components. The system is designed using a centralized architecture, where the ESP32 microcontroller acts as the core processing unit.

The architecture consists of three main layers:

1. Input Layer

The input layer includes user interaction through hardware buttons:

- SOS Button: Used to trigger emergency alerts (short press for SOS, long press for panic mode)

- Call Button: Used to initiate or terminate an emergency call

These inputs are continuously monitored by the ESP32.

2. Processing Layer

The processing layer is handled by the ESP32 microcontroller, which performs the following operations:

- Detects button press type (short press / long press)

- Controls system states such as SOS mode and panic mode

- Communicates with the GPS module to obtain location data

- Processes GPS data to generate a Google Maps link

- Sends commands to the GSM module for SMS and calling

- Controls the buzzer for audio feedback

3. Output Layer

The output layer consists of:

- GSM Module (SIM800L): Sends SMS alerts and makes calls

- Buzzer: Provides audible alerts, especially during panic mode

The system outputs ensure that both remote communication (SMS/call) and local alert (buzzer) are achieved.

IMPLEMENTATION

The implementation of the SOS emergency alert system involves the integration of hardware components with embedded software developed using the Arduino framework on the ESP32 platform. The system is designed to ensure reliable operation of all functionalities, including SOS alert, panic mode, and emergency calling.

4.1 Hardware Implementation

The hardware setup consists of connecting the ESP32 microcontroller with the GPS module, GSM module, push buttons, buzzer, and power supply system.

The NEO-6M GPS module is connected to the ESP32 using UART communication to provide location data. The SIM800L GSM module is also connected via serial communication for sending SMS and making calls using AT commands.

The SOS button and call button are connected to GPIO pins and configured as input devices. The active buzzer is connected as an output device to provide audio alerts. Special attention is given to the power supply design, where a lithium-ion battery is used and a stable supply line is used.

4.2 Software Implementation

The software is developed using C++ in the Arduino IDE, and it controls all system functionalities through structured logic and modular design.

Key Features of Software Implementation:

- Button Press Detection:
The system differentiates between short press and long press of the SOS button using timing logic.

- State Management:
Flags such as panic mode and call status are used to control system behavior.

- GPS Data Processing:
The TinyGPS++ library is used to extract latitude and longitude and convert them into a Google Maps link.

- GSM Communication:
AT commands are used to send SMS and initiate or terminate calls via the SIM800L module.

- Buzzer Control:
Non-blocking logic (using timing functions) is implemented to control the buzzer without interrupting other operations.

- Serial Communication Handling:
Separate communication handling is implemented for GPS and GSM modules to avoid data conflicts.

4.3 System Operation

The working of the system is based on user interaction:

- When the SOS button is pressed briefly, the system sends an emergency SMS along with location details.

- When the SOS button is pressed for a longer duration, the system enters panic mode, sends an alert message, and activates a continuous buzzer.

- When the call button is pressed, the system initiates or ends an emergency call.

The system continuously monitors inputs and ensures that all actions are executed efficiently.

4.4 Challenges and Solutions

During the implementation phase, several challenges were encountered:

- GPS Signal Delay:
The GPS module required time to acquire satellite signals. This was handled by allowing sufficient initialization time.

- GSM Network Instability:
SMS delays and call issues were observed in low-signal areas. Proper timing and retry handling improved reliability.

- Serial Communication Conflicts:
Managing communication between GPS and GSM modules required careful handling to avoid data overlap.

- Firebase Integration Attempt:
Real-time tracking using Firebase was initially attempted. However, due to synchronization issues, network delays, and system instability, it was removed to maintain reliable core functionality.

EXPECTED OUTCOME

The expected outcome of the proposed SOS emergency alert system is to provide a fast, reliable, and user-friendly solution for handling emergency situations.

The system is designed to ensure that users can quickly communicate distress with minimal effort. Upon activation, the device successfully sends an emergency SMS containing the user’s location, allowing the receiver to identify the exact position through a Google Maps link.

The system also enables an emergency calling feature, which allows direct communication between the user and the predefined contact. This ensures that help can be confirmed and coordinated in real time.

In addition, the panic mode feature provides a continuous audible alert through a buzzer, which helps attract attention from nearby people in critical situations.

The device operates independently of internet connectivity, making it reliable in areas with limited network access. Its simple design and hardware-based implementation make it suitable for real-world usage.

Overall, the system is expected to deliver:

- Quick and reliable SOS alert messages with location

- Effective emergency calling capability

- Immediate panic alert through buzzer

- Stable performance without dependency on internet services

APPENDIX A

The appendix section includes additional supporting materials related to the implementation and working of the SOS emergency alert system.

6.1 System Output and Demonstration

This section contains screenshots and images demonstrating the working of the system, including:

- SOS message received on the mobile device

- Google Maps link showing the user’s location

- Panic mode activation with buzzer indication

- Call functionality demonstration

These outputs verify that the system performs all intended operations successfully.

CONCLUDING REMARKS

The SOS Emergency Alert System developed in this project successfully demonstrates a practical and reliable solution for handling emergency situations. By integrating the ESP32 microcontroller with GPS and GSM modules, the system is able to provide quick communication, accurate location sharing, and immediate alert mechanisms.

The project focuses on ensuring simplicity, reliability, and real-world usability. Key features such as SOS messaging, emergency calling, and panic mode with an audible buzzer have been implemented effectively. The system operates independently of internet connectivity, making it suitable for use in areas with limited network access. During the development process, advanced features such as real-time tracking using Firebase were explored. However, due to integration challenges and system instability, these features were not included in the final implementation. Instead, priority was given to building a stable and dependable core system, which is essential in emergency scenarios. Overall, the project achieves its objective of creating a low-cost, portable, and efficient emergency alert device that can be used in real-life situations.

REFERENCES AND BIBLIOGRAPHY

The following resources were referred to during the development of the SOS Emergency Alert System:

- Documentation of the ESP32 Development Board (Espressif Systems)

- NEO-6M GPS Module datasheets and technical references

- SIM800L GSM Module AT Command Manual

- Arduino IDE official documentation

- TinyGPS++ Library documentation and examples

- Online technical forums and tutorials related to embedded systems and IoT development.